# IoTrix Workshop Documentation

## Workshop Information

Team Name:Team Gama

Team Members:

- Ifat Jahan Metu(Team Leader)

- Fariha Lubaba

- Lamia Akter

- Md.Habibur Rahman

- Mehzabeen  Islam

## Objective

The objective of this workshop was to learn the fundamentals of the Internet of Things (IoT), understand the ESP32 development board, set up the Arduino IDE, and gain hands-on experience with IoT programming and circuit simulation.

What is IoT?

The Internet of Things (IoT) is a network of physical devices that are connected to the internet. These devices collect, exchange, and process data automatically using sensors, microcontrollers, and communication technologies like Wi-Fi or Bluetooth.

How IoT Works

An IoT system generally follows these steps:

- Sensors collect data.

- The ESP32 processes the data.

- Data is transmitted through Wi-Fi/Bluetooth.

- Cloud server stores and processes the data.

- Users monitor or control devices using a web or mobile application.

Arduino IDE Setup

### Installation Steps

- Install Arduino IDE.

- Open Board Manager.

- Search ESP32.

- Install ESP32 by Espressif Systems.

- Select ESP32 Dev Module from the board menu.

ESP32 DevKit V1

## Features

- Dual-Core Processor

- Clock Speed: 240 MHz

- Built-in Wi-Fi

- IEEE 802.11 b/g/n

- Bluetooth Support

- Approximately 30 GPIO Pins

- 4 MB Flash Memory

- 520 KB SRAM

### Advantages

- Fast processing

- Low power consumption

- Built-in wireless communication

- Suitable for IoT projects

Equipment Used

- ESP32 DevKit V1

- USB Cable

- Jumper Wires

- LED

- Laptop

- HC-SR04

- DHT11

- Relay Module

- Soil Moisture Sensor

Online Platforms Used

During the workshop, the following online tools were introduced:

- Wokwi (IoT simulation)

- Circuito.io (Circuit Design)

- Cirkit Designer IDE (Circuit Simulation & Design)

GPIO (General Purpose Input/Output)

GPIO pins can be configured as:

- Input

- Output

They are used to read sensor values and control electronic devices like LEDs, relays, and motors.

Digital Signal

Digital signals have only two states:

| Value | Meaning |
| --- | --- |
| 0 | LOW (OFF) |
| 1 | HIGH (ON) |

Example:

- LED OFF → 0

- LED ON → 1

Analog Signal

Analog signals can have many voltage levels.

Example:

0V → Minimum

3.3V (ESP32) → Maximum

Analog values are commonly read from sensors.

### Programming Concepts Learned

- pinMode()

- digitalWrite()

- HIGH

- LOW

- delay()

Task1:

#define LED1  20

#define LED2  19

#define LED3  21

#define LED4  47

#define LED5  48

#define LED6  45

#define LED7  35

#define LED8  36

#define LED9  37

#define LED10 38

void setup() {

pinMode(LED1, OUTPUT);

pinMode(LED2, OUTPUT);

pinMode(LED3, OUTPUT);

pinMode(LED4, OUTPUT);

pinMode(LED5, OUTPUT);

pinMode(LED6, OUTPUT);

pinMode(LED7, OUTPUT);

pinMode(LED8, OUTPUT);

pinMode(LED9, OUTPUT);

pinMode(LED10, OUTPUT);

}

void loop() {

digitalWrite(LED1, HIGH);

digitalWrite(LED2, HIGH);

digitalWrite(LED3, HIGH);

digitalWrite(LED4, HIGH);

digitalWrite(LED5, HIGH);

digitalWrite(LED6, HIGH);

digitalWrite(LED7, HIGH);

digitalWrite(LED8, HIGH);

digitalWrite(LED9, HIGH);

digitalWrite(LED10, HIGH);

delay(500);

digitalWrite(LED1, LOW);

digitalWrite(LED2, LOW);

digitalWrite(LED3, LOW);

digitalWrite(LED4, LOW);

digitalWrite(LED5, LOW);

digitalWrite(LED6, LOW);

digitalWrite(LED7, LOW);

digitalWrite(LED8, LOW);

digitalWrite(LED9, LOW);

digitalWrite(LED10, LOW);

delay(500);

}

Task 2:

#define TRIG_PIN 5

#define ECHO_PIN 18

#define LED_PIN 2

void setup() {

Serial.begin(115200);

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(LED_PIN, OUTPUT);

}

void loop() {

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

long duration = pulseIn(ECHO_PIN, HIGH);

float distance = duration * 0.0343 / 2;

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

if (distance <= 10) {

digitalWrite(LED_PIN, HIGH);

} else {

digitalWrite(LED_PIN, LOW);

}

delay(500);

}

Task 3:

#define SOIL_PIN 34   // Analog Pin

void setup() {

Serial.begin(115200);

}

void loop() {

int sensorValue = analogRead(SOIL_PIN);

Serial.print("Soil Moisture Value: ");

Serial.println(sensorValue);

// Adjust thresholds based on your sensor calibration

if (sensorValue > 3000) {

Serial.println("Soil is DRY");

}

else if (sensorValue > 1800) {

Serial.println("Soil is MOIST");

}

else {

Serial.println("Soil is WET");

}

Serial.println("-------------------");

delay(1000);

}

Task 3.2:

#define SOIL_PIN 34

#define LED_PIN 2

void setup() {

Serial.begin(115200);

pinMode(LED_PIN, OUTPUT);

}

void loop() {

int sensorValue = analogRead(SOIL_PIN);

// Sensor Value → Moisture Percentage

int moisture = map(sensorValue, 4095, 1500, 0, 100);

// Limit between 0 and 100

moisture = constrain(moisture, 0, 100);

Serial.print("Moisture: ");

Serial.print(moisture);

Serial.println("%");

if (moisture > 50) {

digitalWrite(LED_PIN, HIGH);   // LED ON

Serial.println("LED ON");

}

else {

digitalWrite(LED_PIN, LOW);    // LED OFF

Serial.println("LED OFF");

}

delay(1000);

}

Task 3.3:

#define SOIL_PIN 34

#define LED_PIN 2

void setup() {

Serial.begin(115200);

pinMode(LED_PIN, OUTPUT);

}

void loop() {

int sensorValue = analogRead(SOIL_PIN);

int moisture = map(sensorValue, 4095, 1500, 0, 100);

moisture = constrain(moisture, 0, 100);

Serial.print("Moisture: ");

Serial.print(moisture);

Serial.println("%");

if (moisture < 60) {

Serial.println("Moisture LOW - LED Blinking");

digitalWrite(LED_PIN, HIGH);

delay(500);

digitalWrite(LED_PIN, LOW);

delay(500);

}

else {

Serial.println("Moisture OK - LED OFF");

digitalWrite(LED_PIN, LOW);

delay(1000);

}

}

Task 4:

#include <DHT.h>

#define DHTPIN 4

#define DHTTYPE DHT11

#define LED_PIN 2

DHT dht(DHTPIN, DHTTYPE);

unsigned long previousRead = 0;

unsigned long previousBlink = 0;

const unsigned long readInterval = 3500;

unsigned long blinkInterval = 1000;

float humidity = 0;

float temperature = 0;

bool alert = false;

bool ledState = LOW;

void setup() {

Serial.begin(115200);

dht.begin();

pinMode(LED_PIN, OUTPUT);

digitalWrite(LED_PIN, LOW);

Serial.println("DHT11 Sensor Started");

}

void loop() {

unsigned long currentMillis = millis();

if (currentMillis - previousRead >= readInterval) {

previousRead = currentMillis;

humidity = dht.readHumidity();

temperature = dht.readTemperature();

if (isnan(humidity) || isnan(temperature)) {

Serial.println("No DHT11 Detected!");

alert = false;

digitalWrite(LED_PIN, LOW);

return;

}

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" °C");

Serial.print("Humidity: ");

Serial.print(humidity);

Serial.println(" %");

Serial.println("---------------------");

if (humidity > 80 && temperature > 30) {

alert = true;

blinkInterval = 1000;

}

else if (humidity > 70 && temperature > 30) {

alert = true;

blinkInterval = 500;

Serial.println("Warning!");

}

else if (humidity < 70 && temperature < 30) {

alert = false;

digitalWrite(LED_PIN, LOW);

Serial.println("All Good");

}

else {

alert = false;

digitalWrite(LED_PIN, LOW);

}

}

// LED Blink

if (alert) {

if (currentMillis - previousBlink >= blinkInterval) {

previousBlink = currentMillis;

ledState = !ledState;

digitalWrite(LED_PIN, ledState);

}

}

}

Task 5:

/*

==============================================================================

🎯 HACKATHON STARTER TEMPLATE: ESP32 Local Web Server (DHT11 + Water Sensor)

==============================================================================

📌 Microcontroller: ESP32 Dev Module / ESP32 WROOM

📦 Required Libraries (Install via Arduino Library Manager):

1. "DHT sensor library" by Adafruit

2. "Adafruit Unified Sensor" by Adafruit

(Built-in libraries used: <WiFi.h>, <WebServer.h>)

🔌 Hardware Wiring Guide for ESP32:

-------------------------------------------------------------------------

Sensor Pin               | ESP32 Pin     | Notes

-------------------------------------------------------------------------

[DHT11 Temperature & Humidity]

- VCC                    | 3.3V or 5V    | Power (+)

- GND                    | GND           | Ground (-)

- DATA / OUT             | GPIO 4        | Digital Data Pin

[Water Level Sensor]

- VCC / (+)              | 3.3V or 5V    | Power (+)

- GND / (-)              | GND           | Ground (-)

- S / OUT (Signal)       | GPIO 34       | Must use ADC1 (GPIO 34, 35, 36, 39)

-------------------------------------------------------------------------

==============================================================================

*/

#include <WiFi.h>

#include <WebServer.h>

#include "DHT.h"

// ------------------------------------------------------------------------------

// ⚙️ CONFIGURATION SECTION [MODIFIED BY PARTICIPANTS]

// ------------------------------------------------------------------------------

// [TODO 1]: Wi-Fi Network Credentials (ESP32 connects to your local router)

#define WIFI_SSID     "Fariha Lubaba"

#define WIFI_PASSWORD "12345678"

// [TODO 2]: Hardware Pin Configuration

#define DHTPIN           4        // GPIO pin for DHT11 data

#define DHTTYPE          DHT11    // Sensor type

#define WATER_SENSOR_PIN 34       // Analog ADC1 pin for Water Level (GPIO 34)

const int WATER_THRESHOLD = 500;  // Threshold for water detection alert

// ------------------------------------------------------------------------------

// 📦 GLOBAL OBJECTS & INITIALIZATION

// ------------------------------------------------------------------------------

DHT dht(DHTPIN, DHTTYPE);

WebServer server(80); // Web server running on HTTP standard port 80

// Global variables to store sensor readings

float temperature = 0.0;

float humidity = 0.0;

int rawWaterLevel = 0;

float waterPercentage = 0.0;

bool waterDetected = false;

// ------------------------------------------------------------------------------

// 🎨 HTML & CSS DASHBOARD TEMPLATE (Stored in Flash Memory)

// ------------------------------------------------------------------------------

const char HTML_DASHBOARD[] PROGMEM = R"rawliteral(

<!DOCTYPE html>

<html lang="en">

<head>

<meta charset="UTF-8">

<meta name="viewport" content="width=device-width, initial-scale=1.0">

<title>ESP32 Local Sensor Dashboard</title>

<style>

* { box-sizing: border-box; margin: 0; padding: 0; font-family: 'Segoe UI', Tahoma, Geneva, Verdana, sans-serif; }

body { background: #0f172a; color: #f8fafc; display: flex; flex-direction: column; align-items: center; min-height: 100vh; padding: 20px; }

header { margin-top: 20px; text-align: center; }

header h1 { font-size: 2rem; color: #38bdf8; margin-bottom: 5px; }

header p { color: #94a3b8; font-size: 0.95rem; }

.status-badge { background: #1e293b; border: 1px solid #334155; padding: 6px 14px; border-radius: 20px; font-size: 0.85rem; color: #4ade80; display: inline-block; margin-top: 10px; }

.grid-container { display: grid; grid-template-columns: repeat(auto-fit, minmax(250px, 1fr)); gap: 20px; width: 100%; max-width: 900px; margin-top: 30px; }

.card { background: #1e293b; border: 1px solid #334155; border-radius: 16px; padding: 25px; text-align: center; transition: transform 0.2s, border-color 0.2s; box-shadow: 0 10px 25px -5px rgba(0,0,0,0.3); }

.card:hover { transform: translateY(-5px); border-color: #38bdf8; }

.card .icon { font-size: 2.5rem; margin-bottom: 10px; }

.card .label { font-size: 0.9rem; color: #94a3b8; text-transform: uppercase; letter-spacing: 1px; }

.card .value { font-size: 2.5rem; font-weight: bold; margin: 10px 0; color: #f8fafc; }

.card .unit { font-size: 1.2rem; color: #94a3b8; }

.alert-card { grid-column: 1 / -1; background: #1e293b; border: 1px solid #334155; }

.alert-box { padding: 15px; border-radius: 10px; font-weight: bold; text-align: center; }

.alert-dry { background: rgba(74, 222, 128, 0.1); color: #4ade80; border: 1px solid rgba(74, 222, 128, 0.3); }

.alert-wet { background: rgba(248, 113, 113, 0.15); color: #f87171; border: 1px solid rgba(248, 113, 113, 0.4); animation: pulse 1.5s infinite; }

@keyframes pulse { 0% { opacity: 1; } 50% { opacity: 0.6; } 100% { opacity: 1; } }

footer { margin-top: auto; padding: 20px 0; color: #64748b; font-size: 0.85rem; }

</style>

</head>

<body>

<header>

<h1>⚡ ESP32 Local Server Dashboard</h1>

<p>Live Real-Time Sensor Telemetry</p>

<div class="status-badge">● System Online</div>

</header>

<div class="grid-container">

<!-- Temperature Card -->

<div class="card">

<div class="icon">🌡️</div>

<div class="label">Temperature</div>

<div class="value"><span id="temp">--</span><span class="unit"> °C</span></div>

</div>

<!-- Humidity Card -->

<div class="card">

<div class="icon">💧</div>

<div class="label">Humidity</div>

<div class="value"><span id="humidity">--</span><span class="unit"> %</span></div>

</div>

<!-- Water Level Card -->

<div class="card">

<div class="icon">🌊</div>

<div class="label">Water Depth Level</div>

<div class="value"><span id="water">--</span><span class="unit"> %</span></div>

</div>

<!-- Water Detection Alert Card -->

<div class="card alert-card">

<div id="alertBox" class="alert-box alert-dry">Checking Water Sensor Status...</div>

</div>

</div>

<footer>ESP32 Local Web Server | Hackathon Starter Kit</footer>

<script>

// Automatic Live Data Update using AJAX/JSON API fetch

function updateSensorData() {

fetch('/api/data')

.then(response => response.json())

.then(data => {

document.getElementById('temp').innerText = data.temperature.toFixed(1);

document.getElementById('humidity').innerText = data.humidity.toFixed(1);

document.getElementById('water').innerText = data.water_percentage.toFixed(1);

const alertBox = document.getElementById('alertBox');

if (data.water_detected) {

alertBox.innerText = '🚨 WARNING: WATER DETECTED!';

alertBox.className = 'alert-box alert-wet';

} else {

alertBox.innerText = '✅ NORMAL: DRY ENVIRONMENT';

alertBox.className = 'alert-box alert-dry';

}

})

.catch(err => console.error("Error fetching data:", err));

}

// Refresh telemetry every 2 seconds

setInterval(updateSensorData, 2000);

updateSensorData();

</script>

</body>

</html>

)rawliteral";

// ------------------------------------------------------------------------------

// 🛠️ SENSOR READING HELPER

// ------------------------------------------------------------------------------

void readSensors() {

float t = dht.readTemperature();

float h = dht.readHumidity();

if (!isnan(t)) temperature = t;

if (!isnan(h)) humidity = h;

rawWaterLevel = analogRead(WATER_SENSOR_PIN);

waterPercentage = (rawWaterLevel / 4095.0) * 100.0;

waterDetected = (rawWaterLevel > WATER_THRESHOLD);

}

// ------------------------------------------------------------------------------

// 🌐 WEB SERVER ROUTE HANDLERS

// ------------------------------------------------------------------------------

// Handler for Main HTML Web Page Route ("/")

void handleRoot() {

readSensors();

server.send(200, "text/html", HTML_DASHBOARD);

}

// Handler for JSON Data API Endpoint ("/api/data")

void handleApiData() {

readSensors();

String json = "{";

json += "\"temperature\":" + String(temperature, 1) + ",";

json += "\"humidity\":" + String(humidity, 1) + ",";

json += "\"raw_water\":" + String(rawWaterLevel) + ",";

json += "\"water_percentage\":" + String(waterPercentage, 1) + ",";

json += "\"water_detected\":" + String(waterDetected ? "true" : "false");

json += "}";

server.send(200, "application/json", json);

}

// Handler for unknown 404 routes

void handleNotFound() {

server.send(404, "text/plain", "404: Route Not Found");

}

// ------------------------------------------------------------------------------

// 🚀 SETUP FUNCTION

// ------------------------------------------------------------------------------

void setup() {

Serial.begin(115200);

delay(500);

Serial.println();

Serial.println("==================================================");

Serial.println("🌐 ESP32 Local Web Server (DHT11 + Water Sensor)");

Serial.println("==================================================");

// Initialize sensors

dht.begin();

pinMode(WATER_SENSOR_PIN, INPUT);

// Connect to Local Wi-Fi Network

Serial.print("📶 Connecting to Wi-Fi: ");

Serial.println(WIFI_SSID);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println();

Serial.println("✅ Wi-Fi Connected Successfully!");

Serial.print("📍 Open Browser at: http://");

Serial.println(WiFi.localIP());

// Setup Web Server Routes

server.on("/", handleRoot);            // Main Dashboard page

server.on("/api/data", handleApiData); // JSON API endpoint for live JavaScript updates

server.onNotFound(handleNotFound);

// Start HTTP Server

server.begin();

Serial.println("🚀 Local Web Server Started!");

}

// ------------------------------------------------------------------------------

// 🔁 MAIN LOOP FUNCTION

// ------------------------------------------------------------------------------

void loop() {

// Keep listening for incoming HTTP requests from web browsers

server.handleClient();

}